ID office, home intelligent lock

user manual

- 简介 I. Introduction

- Office, home intelligent security lock is my computer management company in the former hotel developed on the basis of the locks out of yet another intelligent lock. 其最大特点是安全、方便、实用。 The greatest feature of a safe, convenient and practical. 全锁的内部机械与电子部分与酒店型完全一样，不同的是控制方法的改变，使安装和维护都更加方便，其保密性同IC卡酒店门锁一样，锁匙不可复制。 Whole-house mechanical and electronic locks and hotel part of the same type, different control methods is the change, so that is more convenient to install and maintain their privacy, like with the IC card hotel lock, keys can not be copied.

- Office, home door lock system, abolition of the hotel complex feature set and operation steps, replaced by a simple and convenient set of RF smart cards, use more convenient.

办公室、家用门锁系统的钥匙有以下两种：  Office, home door lock system, the keys have the following two:

1、主人卡(管理卡) 功能：开门（包括能开反锁）、配置钥匙卡。 1, the master card (management card) function: open (including able to open locked), configure the key card.

2、钥匙卡 功能：开门（不能开反锁）。 2, key card functions: open (not open locked). 最多可配768张副卡。 Vice-up with 768 cards.

II. technical parameters

读卡方法：非接触 Reader method: Non-contact

电源： 6VDC，四节5号高能碱性电池  待机电流：小于25uA，待机时间为1.5年  Power: 6VDC, 4, 5 high-energy alkaline batteries Standby current: less than 25uA, standby time of 1.5 years

工作电流：5  40mA(电机除外) 感应距离：30mm +5 mm    环境温度：0  70 0 C  Current: 5  40mA (excluding motor) sensing distance: 30mm +5 mm Ambient temperature: 0  70 0 C

三、设置主人卡（管理卡） III. set the master card (Management Card)

扭动机械钥匙开门，不要松开，6秒内靠卡，蓝灯亮，蜂鸣器'嘀'一声，即可配制出主人卡。   Twisting machine keys opened the door, do not let go, 6 seconds by card, blue lights, buzzer 'tick' your voice can be the preparation of a master card.

重新设置主人卡时，原来的主人卡全部失效；   Reset the master card, the original owner card all the failure;

四、配置钥匙卡 IV.configuration, Key Card

在门锁的感应区靠一下主人卡，蓝灯亮，蜂鸣器'嘀'一声，门打开，压下手柄，灯亮的同时再靠一下主人卡，红灯和蓝灯同时亮，此时再靠其它的卡片，即可配成副卡，最多可配768张副卡。   In the door of the sensing area by clicking the master card, blue lights, buzzer 'tick' soon as the door opened and pushed down the handle, lights at the same time to rely on what the master card, bright red and blue light at the same time, this time to rely on Other cards, can be dubbed Vice card, with up to 768 deputy cards. 10秒后系统自动取消配置副卡功能。 10 seconds after the system configuration, Vice card features automatically canceled.

设置开门级别和权限：不同级别的人可以开指定的门锁。 Ⅴ. set the open levels and permissions:

- different levels of people can open the specified locks. 如：经理卡可以开指定普通员工的门，但普通员工卡不能开经理的门。 Such as: manager, cards can open the door of the designated general staff, but the general manager of employee card can not open the door.

经理卡： Manager Card:

一般设定为对所有门锁都能打开的卡类，适宜于高级管理人员如总裁、总经理、主管、房主等。  General Settings for all the locks can be opened cards, suitable for senior management personnel such as president and general manager, supervisor, owner and so on. 配置方法：在所有的门锁上，靠一下主人卡，蓝灯亮，蜂鸣器'嘀'一声，门打开，再靠一下主人卡，红灯和蓝灯同时亮，再靠待配的卡片，此卡即配成副卡。 Configuration Method: In all of the locks on the master card by clicking, blue lights, buzzer 'tick' soon as the door opened, and then rely on what the master card, the red and blue light at the same time, bright, and then seek to be equipped with the card, Vice card this card that is dubbed.

普通员工卡： General Staff Card:

一般设定为对指定门锁起作用的卡类，适用于员工，用户或一般行政人员。   General Settings for the role of the designated door cards for employees, users or the general administrative staff. 配置方法：在指定的门锁上，靠一下主人卡，蓝灯亮，蜂鸣器'嘀'一声，门打开，再靠一下主人卡，红灯和蓝灯同时亮，再靠待配的卡片，此卡即可配成普通员工卡。 Configuration Method: In the specified lock on the master card by clicking, blue lights, buzzer 'tick' soon as the door opened, and then rely on what the master card, the red and blue light at the same time, bright, and then seek to be equipped with the card, This card, dubbed the rank card.

六、开门 Ⅵ. open the door

1、主人卡开门 1, the master card door

用主人卡靠近感应区，蓝灯亮，伴随着蜂鸣器“嘀”的一声，提示锁已开。   Near the sensing area with the master card, blue lights, along with the buzzer, "tick" is heard, suggesting that the lock has been opened.

2、钥匙卡开门 2, key card door

用有效的钥匙卡靠近感应区，蓝灯亮，伴随着蜂鸣器“嘀”的一声，提示锁已开。   With a valid key card near the sensor area, blue lights, along with the buzzer, "tick" is heard, suggesting that the lock has been opened.

3、反锁 3, locked

门锁处于反锁时，只有主人卡可以开锁，普通钥匙卡不可以打开反锁门。   When the door is locked, only the master card can unlock the ordinary key card can not open the locked door.

七、注销开门副卡 Ⅶ. write-offs, Vice card door

1、主人卡靠1次开门，再靠1次红蓝灯交替闪烁，再靠1次亮红灯并“嘀”一声后即可删除所有的开门副卡。   the master card through a second door, and then seek a second alternating red and blue lights flashing, and then seek a second red flag, and "tick" soon as you can delete all of the door after the Vice-cards.

八． Ⅷ. 通道功能 Channel function

靠卡开门后，压下室内手柄（或斜舌）的同时再靠一下开门卡，门锁处于通道状态，再靠卡取消通道功能。   By card opened the door and pushed down the interior handle (or oblique tongue) and then by about the same time open the door cards, door locks in the channel status, and then seek card abolished channel function.